- Floor Doors - Guide Specifications

- SECTION 08 31 16          (1995 - 08315)

- PART 1: GENERAL

1.01    RELATED DOCUMENTS

A.     Conditions of the Contract and portions of Division One of this Project Manual apply to this Section as though repeated herein.

- 1.02    SUMMARY

Select one or more of the following subparagraphs as required by the Project.

A.     Section Includes:

1.      Aluminum Angle [Single] [Double] Floor Door.

2.      Aluminum Gutter [Single] [Double] Floor Door.

3.      Aluminum Recessed [Single] [Double] Floor Door.

- 1.03    SUBMITTALS

A.  Product Data: For each type of floor door indicated.  Include manufacturer's model number and accessories and requirements for roughing-in dimensions.

- 1.04    DELIVERY, STORAGE AND HANDLING

A.     Packing, Shipping, Handling and Unloading: Deliver floor door as a factory assembled unit in manufacturer’s original packaging.

B.    Store materials in a dry, protected, well-vented area.  The contractor shall thoroughly inspect product upon receipt and report damaged material immediately to delivering carrier and note such damage on the carrier’s freight bill of lading.

- PART 2: PRODUCTS

2.01    MANUFACTURER

A.   J L Industries, Incorporated

4450 West 78th Street Circle

- Bloomington, MN 55435

2.02    MANUFACTURED UNITS

A.  Aluminum Angle Floor Door.

Select one or more of the following for each aluminum angle floor door size. If more than one size is required for the project, identify each type on a schedule located on the drawings and identify each type in the specification section. The first 8 options are for single door units, the next 4 options are for double door units.

1.   Model: AAF-2424S; 23 inches by 23 inches clear opening, single door.

2.   Model: AAF-2436S; 23 inches by 35 inches clear opening, single door.

3.   Model: AAF-3030S; 29 inches by 29 inches clear opening, single door.

4.   Model: AAF-3036S; 29 inches by 35 inches clear opening, single door.

5.   Model: AAF-3636S; 35 inches by 35 inches clear opening, single door.

6.   Model: AAF-3048S; 29 inches by 47 inches clear opening, single door.

7.   Model: AAF-3648S; 35 inches by 47 inches clear opening, single door.

8.   Model: AAF-4242S; 41 inches by 41 inches clear opening, single door.

9.   Model: AAF-4848D; 47 inches by 47 inches clear opening, double door.

10. Model: AAF-4860D; 47 inches by 59 inches clear opening, double door.

11. Model: AAF-4872D; 47 inches by 71 inches clear opening, double door.

12. Model: AAF-6060D; 59 inches by 59 inches clear opening, double door.

13. Characteristics:

a.   Door Frames: Extruded aluminum.

b.   Door: 1/4 inch diamond plate aluminum.

c.   Hinges: Stainless steel.

d.   Fasteners: Tamper proof stainless steel bolts.

e.   Hardware: Stainless steel slam locks, gas assist operators, hold open arm, and bituminous coating.

f.    Live Load: 300 psf.

B.  Aluminum Gutter Floor Door.

Select one or more of the following for each aluminum gutter floor door size. If more than one size is required for the project, identify each type on a schedule located on the drawings and identify each type in the specification section. The first 8 options are for single door units, the next 4 options are for double door units.

1.   Model: AGF-2424S; 20-1/2 inches by 20-1/2 inches clear opening, single door.

2.   Model: AGF-2436S; 20-1/2 inches by 32-1/2 inches clear opening, single door.

3.   Model: AGF-3030S; 26-1/2 inches by 26-1/2 inches clear opening, single door.

4.   Model: AGF-3036S; 26-1/2 inches by 32-1/2 inches clear opening, single door.

5.   Model: AGF-3636S; 32-1/2 inches by 32-1/2 inches clear opening, single door.

6.   Model: AGF-3048S; 26-1/2 inches by 44-1/2 inches clear opening, single door.

7.   Model: AGF-3648S; 32-1/2 inches by 44-1/2 inches clear opening, single door.

8.   Model: AGF-4242S; 38-1/2 inches by 38-1/2 inches clear opening, single door.

9.   Model: AGF-4848D; 44-1/2 inches by 44-1/2 inches clear opening, double door.

10. Model: AGF-4860D; 44-1/2 inches by 56-1/2 inches clear opening, double door.

11. Model: AGF-4872D; 44-1/2 inches by 68-1/2 inches clear opening, double door.

12. Model: AGF-6060D; 56-1/2 inches by 56-1/2 inches clear opening, double door.

13. Characteristics:

a.   Door Frames: Extruded aluminum.

b.   Door: 1/4 inch diamond plate aluminum.

c.   Hinges: Stainless steel.

d.   Gutter: Equipped with 1-1/2 inch FPT drain coupling.

e.   Fasteners: Tamper proof stainless steel bolts.

f.    Hardware: Stainless steel slam locks, gas assist operators, hold open arm, and bituminous coating.

g.   Live Load: 300 psf.

C.  Aluminum Recessed Floor Door.

Select one or more of the following for each aluminum angle floor door size. If more than one size is required for the project, identify each type on a schedule located on the drawings and identify each type in the specification section. The first 5 options are for single door units, the next 4 options are for double door units. These units are designed to accept carpet or composition material.

1.   Model: RFD-1; 24 inches by 24 inches clear opening, single door.

2.   Model: RFD-2; 30 inches by 30 inches clear opening, single door.

3.   Model: RFD-3; 30 inches by 36 inches clear opening, single door.

4.   Model: RFD-4; 36 inches by 36 inches clear opening, single door.

5.   Model: RFD-5; 42 inches by 42 inches clear opening, single door.

6.   Model: RFDD-1; 48 inches by 48 inches clear opening, single door.

7.   Model: RFDD-2; 48 inches by 72 inches clear opening, single door.

8.   Model: RFDD-3; 60 inches by 60 inches clear opening, single door.

9.   Model: RFDD-4; 60 inches by 84 inches clear opening, double door.

13. Characteristics:

a.   Frames: 1/4 inch extruded aluminum with 1-1/8 inch anchor straps bolted to frame.

b.   Floor Plate: 1/4 inch flat plate aluminum with 1/4 inch flat aluminum reinforcing bars welded to plate frame.

c.   Fasteners: Tamper proof stainless steel bolts.

d.   Hardware: Heavy stamped 1/4 inch aluminum hinge, tension bars, one-point latch, inside and outside handles, and automatic hold open with vinyl grip.

e.   Live Load: 150 psf.

- PART 3: EXECUTION

3.01    EXAMINATION

A.  Examine areas and conditions affecting installation.  Do not proceed with installation until unsatisfactory conditions have been corrected.

3.02    INSTALLATION

A.  Install system level and plumb, and in accord with manufacturer’s written instructions, original design and referenced standards.

- 3.03    ADJUSTING

A.  Adjust system for proper operation. Replace any parts that prevent the floor door from operating properly.

3.04    CLEANING

A.  Remove all debris caused by installation of the floor door. Clean all exposed surfaces to as fabricated condition and appearance.

3.05    PROTECTION

A.  Provide protection of the completed installation until completion of the project. Repair any damage at no additional cost.

3.06    SCHEDULES

If more than one type of floor door is required, provide a schedule similar to the following example below. Use the corresponding type designations here and on the drawings,

A.     Type 1: [Describe by model].

B.    Type 2: [Describe by model].

C.    Type 3: [Describe by model